国内外同类方法参数量与控制效果对比调研

一、PAC 规格（基准）

| 指标 | 数值 |
| --- | --- |
| Transformer 骨干 | 14,113 参数 / 55.1 KB (FP32) |
| 残差头 | 2,177 参数 / 8.5 KB |
| 部署总计 | 16,290 参数 / 63.6 KB |
| 每步推理代价 | ~0.23M MACs |
| 输出维度 | 1（标量 α） |
| 控制频率 | 100 Hz |
| ID RMSE | 0.066 m |
| OOD RMSE | 0.116 m（−28% vs 监督基线） |
| 执行器退化 RMSE | 0.090 m（MPC 单独：0.822 m） |

二、对标论文对比

| 方法 | 来源 | 参数量 | 模型大小 | 输出维度 | 安全回退 | OOD测试 |
| --- | --- | --- | --- | --- | --- | --- |
| PAC（本文） | — | 16,290 | 63.6 KB | 1（α） | ✓ 零初始化 | 10族 |
| Cai et al. | ICRA 2025 | ~70K* | ~280 KB* | 6（推杆） | ✗ | 参数偏移 |
| Fan et al. | TCST 2024 | ~320K* | ~1.3 MB* | 6（推杆） | ✗ | 无 |
| Johannink et al. | ICRA 2019 | ~100K* | ~400 KB* | 关节空间 | ✗ | 无 |
| Xu et al. RBF-NN | EAAI 2025 | 100–500** | <2 KB | 控制律 | Lyapunov | 无 |
| TinyMPC | ICRA 2024 | N/A（非NN） | 10–50 KB | 6（推杆） | 数学保证 | 无 |
| Decision Trans. | NeurIPS 2021 | 1M–5M | 4–20 MB | 动作序列 | ✗ | 无 |

* 参数量为根据论文描述的架构和标准配置推算的估计值，原论文未明确报告总参数量。这是该领域的普遍问题——2025 年 arXiv 综述（2504.15129）专门指出“学习型端到端控制很少报告实现细节”。

** RBF 自适应 NN 参数极少（5–20 个神经元），但这是在线自适应方法，与离线训练的深度学习方法不可直接比参数量。

三、轻量化排名

在深度学习控制器中，PAC 的 16,290 参数是最小的——为同类 RL 控制器的 1/4 到 1/20。63.6 KB 可直接部署在 ARM Cortex-M 级别微控制器上。

| 方法 | 参数量 | 相对 PAC 倍数 |
| --- | --- | --- |
| RBF 自适应 NN | ~200 | 0.01x |
| PAC（本文） | 16,290 | 1x |
| Cai ICRA 2025 | ~70,000 | 4.3x |
| Johannink ICRA 2019 | ~100,000 | 6.1x |
| Fan TCST 2024 | ~320,000 | 19.6x |
| Decision Transformer | ~1,000,000+ | 61x+ |

四、控制效果定位

1. 绝对精度：ID 0.066 m、OOD 0.116 m。不同论文评估协议不同，直接比米数不公平。

2. OOD 鲁棒性：PAC 是唯一一个在 10 族分布外扰动上做了系统性测试的——Cai 只测了参数偏移，Fan 没有 OOD 实验。

3. 安全回退：没有对比论文提供“最坏=已验证基线”保证。这是部署认证优势。

4. 可解释性：输出 1 维权限 α 可直接审计；端到端输出 6 维推杆指令不可分解。

五、核心结论

PAC 在深度学习控制器中轻量化程度绝对领先（16,290 参数，63.6 KB，0.23M MACs @100 Hz），同时独有提供可验证的最坏情形回退到已验证基线的安全保证。在 10 族分布外扰动上的系统测试（OOD −28% + 执行器退化免疫 0.822→0.090 m）在同类文献中没有对标。

六、参考文献

[1]  L. Cai, K. Chang, and Y. Girdhar. Learning to swim: Reinforcement learning for 6-DOF control of thruster-driven autonomous underwater vehicles. In Proc. IEEE ICRA, 2025.

[2]  Y. Fan, H. Dong, X. Zhao, and P. Denissenko. Path-following control of unmanned underwater vehicle based on an improved TD3 deep reinforcement learning. IEEE Trans. Control Systems Technology, 32(5):1904–1919, 2024.

[3]  T. Johannink et al. Residual reinforcement learning for robot control. In Proc. IEEE ICRA, 2019, pp. 6023–6029.

[4]  K. Nguyen, S. Schoedel, A. Alavilli, B. Plancher, and Z. Manchester. TinyMPC: Model-predictive control on resource-constrained microcontrollers. In Proc. IEEE ICRA, 2024.

[5]  W. Xu. Low complexity adaptive neural network three-dimensional tracking control of autonomous underwater vehicles. Engineering Applications of Artificial Intelligence, 137:109860, 2025.

[6]  L. Chen et al. Decision Transformer: Reinforcement learning via sequence modeling. In NeurIPS, 2021.